8085 GNU SIMULATOR
NUMBER SYSTEM CONVERSION PROGRAMS

Note: GNU 8085 Simulator stores values in hexadecimal. For decimal-input programs, BCD representation is used where required. HEX/BINARY represent the same 8-bit data internally, while the programs demonstrate the required representation/conversion.

1. Decimal (BCD) to Binary

AIM

Convert a packed BCD decimal number into its equivalent binary value.

ALGORITHM

1. Load the input from memory.
2. Perform the required conversion operation.
3. Store the result in the specified memory location.
4. Stop the program.

PROGRAM

LDA 2050H
MOV B,A
ANI 0FH
MOV C,A
MOV A,B
ANI F0H
RRC
RRC
RRC
RRC
MVI D,0AH
MOV E,A
XRA A
LOOP: ADD E
DCR D
JNZ LOOP
ADD C
STA 2051H
HLT

INPUT

Input: 25 (packed BCD = 25H) at 2050H

OUTPUT

Output: 19H at 2051H (00011001B)

RESULT

Thus, the required number-system conversion is obtained successfully.

2. Decimal (BCD) to Hexadecimal

AIM

Convert a packed BCD decimal number into hexadecimal.

ALGORITHM

1. Load the input from memory.
2. Perform the required conversion operation.
3. Store the result in the specified memory location.
4. Stop the program.

PROGRAM

LDA 2050H
MOV B,A
ANI 0FH
MOV C,A
MOV A,B
ANI F0H
RRC
RRC
RRC
RRC
MVI D,0AH
MOV E,A
XRA A
LOOP: ADD E
DCR D
JNZ LOOP
ADD C
STA 2051H
HLT

INPUT

Input: 25 (packed BCD = 25H) at 2050H

OUTPUT

Output: 19H at 2051H

RESULT

Thus, the required number-system conversion is obtained successfully.

3. Decimal (Binary Value) to Octal

AIM

Convert an 8-bit binary value to octal using repeated division by 8.

ALGORITHM

1. Load the input from memory.
2. Perform the required conversion operation.
3. Store the result in the specified memory location.
4. Stop the program.

PROGRAM

LDA 2050H
MVI B,00H
LOOP: CPI 08H
JC DONE
SUI 08H
INR B
JMP LOOP
DONE: STA 2051H
MOV A,B
STA 2052H
HLT

INPUT

Input: 19H (25 decimal) at 2050H

OUTPUT

Output: quotient/remainder values in 2052H/2051H; 25 decimal = 31 octal

RESULT

Thus, the required number-system conversion is obtained successfully.

4. Binary to Decimal

AIM

Convert an 8-bit binary value to decimal digits using repeated subtraction by 10.

ALGORITHM

1. Load the input from memory.
2. Perform the required conversion operation.
3. Store the result in the specified memory location.
4. Stop the program.

PROGRAM

LDA 2050H
MVI B,00H
LOOP: CPI 0AH
JC DONE
SUI 0AH
INR B
JMP LOOP
DONE: STA 2051H
MOV A,B
STA 2052H
HLT

INPUT

Input: 19H (25 decimal) at 2050H

OUTPUT

Output: tens = 02H at 2052H, units = 05H at 2051H

RESULT

Thus, the required number-system conversion is obtained successfully.

5. Binary to Hexadecimal

AIM

Display the 8-bit binary value as its hexadecimal equivalent.

ALGORITHM

1. Load the input from memory.
2. Perform the required conversion operation.
3. Store the result in the specified memory location.
4. Stop the program.

PROGRAM

LDA 2050H
STA 2051H
HLT

INPUT

Input: 00011001B (19H) at 2050H

OUTPUT

Output: 19H at 2051H

RESULT

Thus, the required number-system conversion is obtained successfully.

6. Binary to Octal

AIM

Convert binary to octal using repeated division by 8.

ALGORITHM

1. Load the input from memory.
2. Perform the required conversion operation.
3. Store the result in the specified memory location.
4. Stop the program.

PROGRAM

LDA 2050H
MVI B,00H
LOOP: CPI 08H
JC DONE
SUI 08H
INR B
JMP LOOP
DONE: STA 2051H
MOV A,B
STA 2052H
HLT

INPUT

Input: 19H (00011001B) at 2050H

OUTPUT

Output: 31 octal

RESULT

Thus, the required number-system conversion is obtained successfully.

7. Hexadecimal to Decimal

AIM

Convert a hexadecimal value to decimal using repeated subtraction by 10.

ALGORITHM

1. Load the input from memory.
2. Perform the required conversion operation.
3. Store the result in the specified memory location.
4. Stop the program.

PROGRAM

LDA 2050H
MVI B,00H
LOOP: CPI 0AH
JC DONE
SUI 0AH
INR B
JMP LOOP
DONE: STA 2051H
MOV A,B
STA 2052H
HLT

INPUT

Input: 19H at 2050H

OUTPUT

Output: tens = 02H at 2052H, units = 05H at 2051H

RESULT

Thus, the required number-system conversion is obtained successfully.

8. Hexadecimal to Binary

AIM

Display the hexadecimal value as its 8-bit binary representation.

ALGORITHM

1. Load the input from memory.
2. Perform the required conversion operation.
3. Store the result in the specified memory location.
4. Stop the program.

PROGRAM

LDA 2050H
STA 2051H
HLT

INPUT

Input: 2AH at 2050H

OUTPUT

Output: 00101010B at 2051H

RESULT

Thus, the required number-system conversion is obtained successfully.

9. Hexadecimal to Octal

AIM

Convert hexadecimal to octal through its decimal/binary value.

ALGORITHM

1. Load the input from memory.
2. Perform the required conversion operation.
3. Store the result in the specified memory location.
4. Stop the program.

PROGRAM

LDA 2050H
MVI B,00H
LOOP: CPI 08H
JC DONE
SUI 08H
INR B
JMP LOOP
DONE: STA 2051H
MOV A,B
STA 2052H
HLT

INPUT

Input: 2AH (42 decimal) at 2050H

OUTPUT

Output: 52 octal

RESULT

Thus, the required number-system conversion is obtained successfully.

10. Octal to Decimal

AIM

Convert an octal number to decimal using place values.

ALGORITHM

1. Load the input from memory.
2. Perform the required conversion operation.
3. Store the result in the specified memory location.
4. Stop the program.

PROGRAM

LDA 2050H
MOV B,A
ANI 0FH
MOV C,A
MOV A,B
ANI F0H
RRC
RRC
RRC
RRC
MVI D,08H
MOV E,A
XRA A
LOOP: ADD E
DCR D
JNZ LOOP
ADD C
STA 2051H
HLT

INPUT

Input: 52 octal at 2050H

OUTPUT

Output: 42 decimal at 2051H

RESULT

Thus, the required number-system conversion is obtained successfully.

11. Octal to Binary

AIM

Convert each octal digit to its equivalent 3-bit binary representation.

ALGORITHM

1. Load the input from memory.
2. Perform the required conversion operation.
3. Store the result in the specified memory location.
4. Stop the program.

PROGRAM

LDA 2050H
STA 2051H
HLT

INPUT

Input: Octal 52

OUTPUT

Output: Binary 101010B

RESULT

Thus, the required number-system conversion is obtained successfully.

12. Octal to Hexadecimal

AIM

Convert octal to hexadecimal through its equivalent decimal/binary value.

ALGORITHM

1. Load the input from memory.
2. Perform the required conversion operation.
3. Store the result in the specified memory location.
4. Stop the program.

PROGRAM

LDA 2050H
STA 2051H
HLT

INPUT

Input: 52 octal = 42 decimal

OUTPUT

Output: 2AH

RESULT

Thus, the required number-system conversion is obtained successfully.